PROWORDS - Procedural words or “prowords” are words or phrases that have special meaning to expedite the flow and accuracy of voice communications.  Prowords used in the heading or text of a message, are not counted in the group count and are not written into the message when it is prepared for delivery to the addressee.

PROWORD(S)						EXPLANATION

AFFIRMATIVE			YOU ARE CORRECT.  WHAT YOU HAVE

TRANSMITTED IS CORRECT. YES.

ALL AFTER	THE PORTION OF THE MESSAGE TO WHICH I REFER IS THAT PORTION WHICH FOLLOWS- - - -

ALL BEFORE	THE PORTION OF THE MESSAGE TO WHICH I REFER IS THAT PORTION WHICH PRECEDES- - - -

AUTHENTICATE	SPELL PHONETICALLY THE ORIGINATING STATION’S AUTHENTICATION CODE WHICH APPLIES TO THIS MESSAGE.

BREAK	INDICATES THE SEPARATION OF THE TEXT FROM OTHER PORTIONS OF THE MESSAGES.

CORRECT	YOU ARE CORRECT. WHAT YOU HAVE TRANSMITTED IS CORRECT.

CORRECTION	AN ERROR HAS BEEN MADE IN THIS TRANSMISSION. THE CORRECT VERSION IS- - - - -.

DISREGARD, OUT.			THIS TRANSMISSION IS IN ERROR.

DISREGARD IT.

DO NOT ANSWER	STATIONS CALLED WILL NOT ANSWER THIS CALL OR TRANSMIT REGARDING THIS TRANSMISSION.

FROM	THE ORIGINATOR OF THIS MESSAGE FOLLOWS.

GROUPS	THE TEXT OF THIS MESSAGE CONTAINS (XX) GROUPS.

PROWORD(S)						EXPLANATION

I AUTHENTICATE	AUTHENTICATION CODE FOLLOWS

IMMEDIATE	THIS MESSAGE HAS A PRECEDENCE OF IMMEDIATE

INFO	THE ADDRESSEES FOLLOWING ARE ADDRESSED FOR INFORMATION ONLY.

INITIAL(S)	A GROUP OF ONE OR MORE CHARACTERS, THE FIRST OF WHICH IS A LETTER, FOLLOWS.

I READ BACK	THE FOLLOWING IS A RESPONSE TO YOUR INSTRUCTIONS TO “READ BACK”.

I SAY AGAIN	I AM REPEATING THE TRANSMISSION OR PORTION INDICATED.

I SPELL	I WILL SPELL THE NEXT WORD PHONETICALLY.

I VERIFY	THAT WHICH FOLLOWS IS VERIFIED PER YOUR REQUEST.

MAYDAY (repeat 3x)	I AM IN SERIOUS DANGER AND REQUIRE IMMEDIATE HELP.

MORE TO FOLLOW	I HAVE MORE MESSAGES, TRAFFIC OR INFORMATION FOR YOU.

NEGATIVE				NOT RECEIVED. NO.

NO TRAFFIC			I HAVE NO TRAFFIC TO SEND.

NOTHING HEARD			THE STATION CALLED DID NOT RESPOND.

OUT 	END TRANSMISSION NO ANSWER IS REQUIRED OR EXPECTED.

OVER                               	END OF MY TRANSMISSION, I AM READY TO COPY YOUR REPLY .

PAN     (repeat 3x)  			I HAVE AN URGENT SITUATION,

OR TRAFFIC

PROWORD(S)						EXPLANATION

PRIORITY                MESSAGE HAS PRECEDENCE OF PRIORITY”

READ BACK            REPEAT THIS TRANSMISSION EXACTLY AS RECEIVED.

RELAY (TO)            TRANSMIT THIS MESSAGE TO ALL ADDRESSEES

OR ADDRESSEES IMMEDIATELY FOLLOWING

RELAY THROUGH   RELAY THIS MESSAGE THROUGH CALLSIGN______.

ROGER                    I HAVE RECEIVED AND UNDERSTAND YOUR LAST

TRANSMISSION.

ROUTINE                THIS MESSAGE HAS A PRECEDENCE OF “ROUTINE”.

SAY AGAIN             REPEAT ALL OF YOUR LAST TRANSMISSION.

SECURITE’ 		I HAVE A NAVIGATION PROBLEM OR I AM EXPECTING A

(repeat 3x)		STATE OF EMERGENCY TO DEVELOP OR I HAVE

INFORMATION REGARDING A HAZARD TO NAVIGATION.

SERVICE                THIS IS A SERVICE MESSAGE AND HAS NO GROUP

COUNT.

SEND SLOWER      YOUR TRANSMISSION SPEED IS TOO FAST TO COPY.

REDUCE SPEED.

THIS IS                    THIS TRANSMISSION IS FROM THE STATION WHOSE

CALLSIGN FOLLOWS.

THIS IS A DIRECTED NET          FROM NOW UNTIL FURTHER NOTICE

THIS NET IS DIRECTED

.

THIS IS A FREE NET        FROM NOW UNTIL FURTHER NOTICE THIS

NET IS FREE.

TIME                   	FOLLOWING IS THE TIME OR DATE/TIME/GROUP OF

THIS MESSAGE.

TO                           THE FOLLOWING ADDRESSEE(S) ARE ADDRESSED

FOR ACTION.

PROWORD(S)						EXPLANATION

UNKNOWN STATION       THE IDENTITY OF THE STATION IS UNKNOWN.

IDENTIFY AND SAY AGAIN.

URGENT                THIS MESSAGE HAS A PRECEDENCE OF “URGENT”.

VERIFY                   VERIFY ENTIRE MESSAGE (OR A PORTION

INDICATED) WITH THE ORIGINATOR AND SEND THE CORRECT VERSION.

WAIT                 	I MUST PAUSE FOR A FEW SECONDS.

DO NOT TRANSMIT.

WAIT OUT       	I MUST PAUSE FOR A PERIOD OF TIME. CONTINUE WITH THE NET AND I CALL AGAIN LATER. (does not require an answer).

WILCO                     I HAVE RECEIVED AND WILL COMPLY WITH YOUR

LAST TRANSMISSION. The meaning of “ROGER” is 	included in “WILCO” and the two prowords are never used

together.

WORD AFTER       THE WORD I NEED A FILL OF IS THE WORD AFTER - - - -.

WORD BEFORE     THE WORD I NEED A FILL OF IS THE WORD

PRECEDEING - - - -.

WORDS TWICE      COMMUNICATION IS DIFFICULT. TRANSMIT EACH

GROUP OR PHRASE TWICE.

WRONG                  YOUR LAST TRANSMISSION WAS INCORRECT. THE

CORRECT VERSION IS- - - .

A copy of the ITU phonetic alphabet and this proword list should be kept at the operating position until they are committed to memory.